R.E.T. Report

# Testing of Large Scintillation Counters

# By

# Allen Daniel

# 14 July 2004

## Table of Contents

- Introduction                                                                        2

- Procedure                                                                            2

- Data and Results                                                                 6

- Graphs                                                                                 9

- Problems encountered                                                         11

6     Summary                                                                             11

1 Introduction

A number of acrylic scintillator type particle detectors were recovered from Brookhaven National Laboratory. They consisted of a 91.4cm x 45.7cm x 2.1cm acrylic scintillator with a 5.2cm diameter photomultiplier tube mounted at the end of the light guide. The light guides were glued by Dave Kraus, Julia Thompson and Danny Frankie, prior to the start of the workshop. The purpose of this study is to:

- Test the number of photoelectrons per cosmic ray ratio (PE/CR) as a function of distance along the scintillator before assembly.

- To glue the photomultipliers onto the light guide and the PE/CR ratio as a function of distance remeasured.

- Rewrap the counters and test for light tightness using an oscilloscope.

- Procedure: initial tests

The scintillators were placed in a light tight box and the light guide was attached  through a “cookie” to the photomultiplier. Care was taken to achieve a good seal between the “cookie” and the PMT, so that no air gaps were visible. The PMT tube number 23 and base number 19 were used for these tests in order to have the same one pe gain for all the tests. Tube number 23 was selected because of its good first dynode gain of 7.  The box was tested for light tightness by examining the pulse from the scintillator using an oscilloscope and a flashlight. Details concerning the study of the one PE signal can be found in other reports.

The testing was done by placing a smaller detector named ‘Pids’ on top of the large dark box at distances of 10cm, 40cm, and 80cm corresponding to far, middle and near to the PMT (see figure 1).

#### Figure 1

### The power supply used was of a variable voltage high-tension supply with a potentiometer to select the voltage. Each turn on the potentiometer represents an approximate voltage of 200V.

The measurements were taken using the QVT (Charge, Voltage and Time) portion of the DTAKE electronic module and software. The threshold was around 30mV and it was triggered through a non-inverting isolating transformer.  The trigger rate was 20 counts per second, three times what we would expect for CR. In order to calculate the PE/CR ratio, the following measurements were taken.

- One Photoelectron energy peak: it was measured by using the self- trigger function of the QVT. Using this function makes the one PE peak dependent only on the PMT (Phomultiplier tube). To check for stability the one PE was taken on a regular basis.

- The Cosmic ray peak: to find the Cosmic ray peak the external trigger function of the QVT was used. The smaller counter (Pids) was triggered and the number of coincidence counts was measured at the lower larger counter. The cosmic ray peak was identified by inspection, and sometimes the “fit gaussian” utility of the QVT program was employed.

- The pedestal energy peak: this is a trickle of charge which the ADC (Analog to Digital Converter) adds to each channel or bin as a calibration. It is used as an offset to which all other measurements must be referenced.

- A Histogram was created by the QVT program with the X-axis being the bin numbers where each bin is equal to 2.5 x 10-13 Coulombs of charge.

- With the information gathered the PE/CR ratio was calculated. The calculation was performed using the formula PECR=(CR−Ped)CR(PE−Ped)PE where CR, PE and Ped represent the cosmic ray, Photoelectron, and Pedestal measurements respectively.

##### Data and Results

Data table one provides a summary of the PE/CR ratio as a function of distance along UMSL8A detector under various conditions.

|  | Run# | Date / Time | Distance (cm) | Ped (Bin) | One Pe (Bin) | CR (Bin) | PE/CR | Ratio of Near(80cm) to far (10cm) | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8A | 22 | 6/30/04 | 10 | 28 | 38 | 70 | 4 |  |  |  |
| 8A | 23 | 6/30/04 | 40 | 28 | 38 | 109 | 8 |  | 6dB att. |  |
| 8A | 24 | 6/20/04 | 80 | 28 | 38 | 370 | 34 |  | Elizabeth Langford's power supply was used for runs 22 to 49. Voltage set at 1775V |  |
| 8A | 29 | 6/20/04 | 10 | 28 | 38 | 64 | 4 |  | PMT tube reseated |  |
| 8A | 30 | 6/20/04 | 40 | 28 | 38 | 117 | 9 | 9 |  |  |
| 8A | 31 | 6/20/04 | 80 | 28 | 38 | 401 | 37 |  |  |  |
| 8A | 35 | 6/20/04 | 10 | 28 | 38 | 67 | 4 |  | Washed with distilled water and alcohol |  |
| 8A | 36 | 6/20/04 | 40 | 28 | 38 | 120 | 9 | 9 |  |  |
| 8A | 37 | 6/20/04 | 80 | 28 | 38 | 366 | 34 |  |  |  |
| 8A | 40 | 6/20/04 | 10 | 28 | 38 | 75 | 5 |  | Al Mylar taped on at the end of scintillator |  |
| 8A | 41 | 6/20/04 | 40 | 28 | 38 | 107 | 8 | 8 |  |  |
| 8A | 43 | 6/20/04 | 80 | 28 | 38 | 420 | 39 |  |  |  |
| 8A | 47 | 6/20/04 | 10 | 28 | 38 | 71 | 4 |  | Loose collar put on light guide |  |
| 8A | 48 | 6/20/04 | 40 | 28 | 38 | 117 | 9 | 9 |  |  |
| 8A | 49 | 6/20/04 | 80 | 28 | 38 | 390 | 36 |  |  |  |

#### Table 1: PE/CR ratio for UMSL8A

From the PE/CR coulmn of the table we can see that there was no significant decrease in the ratio far/near.

Data table two summarizes the results of PE/CR ratio for large counters 7A to 12C

|  | Run # |  | Date / Time | Distance (cm) | Ped (Bin) | One Pe (bin) | CR | Pe/CR | Ratioof near(80cm) to far 10(cm) | Turns | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7A | 101 |  | 7/6/04 |  |  | 29 |  |  |  | 8.5 |  |
| 7A | 102 |  | 7/6/04 | 10 | 16 | 29 | 75 | 5 |  | 8.5 |  |
| 7A | 103 |  | 7/6/04 | 40 | 16 | 29 | 139 | 10 | 6 | 8.5 |  |
| 7A | 104 |  | 7/6/04 | 80 | 16 | 29 | 390 | 29 |  | 8.5 |  |
| 7B | 85 |  | 7/6/04 | 10 | 14 | 21 | 53 | 5 |  | 8 | Changing HV |
| 7B | 86 |  | 7/6/04 |  | 14 | 21 |  |  |  | 8 |  |
| 7B | 87 |  | 7/6/04 | 40 | 14 | 21 | 74 | 9 | 11 | 8 |  |
| 7B | 88 |  | 7/6/04 | 80 | 14 | 21 | 210 | 28 |  | 8 |  |
| 7B | 89 |  | 7/6/04 |  |  | 28 |  |  |  | 8.5 |  |
| 7B | 90 |  | 7/7/04 | 10 | 14 | 28 | 95 | 6 |  | 8.5 |  |
| 7B | 91 |  | 7/7/04 |  | 16 | 29 |  |  |  | 8.5 |  |
| 7B | 93 |  | 7/7/04 | 10 | 16 | 21 | 48 | 3 |  | 8 | Mylar at the end. Only small improvement |
| 7B | 94 |  | 7/7/04 7/7/04 | 40 | 16 | 29 | 68 | 4 |  | 8.5 |  |
| 7B | 95 |  | 7/7/04 | 80 | 16 | 29 | 457 | 34 |  | 8.5 |  |
| 7B | 96 |  | 7/7/04 |  | 16 | 28 |  |  |  | 8.5 |  |
| 7C | 121 |  | 7/7/04 11:33 |  |  | 27 |  |  |  | 8.5 |  |
| 7C | 122 |  | 7/7/04 11:36 | 10 | 16 | 27 | 96 | 7 |  | 8.5 |  |
| 7C | 123 |  | 7/7/04 11:46 | 40 | 16 | 27 | 140 | 11 | 5 | 8.5 |  |
| 7C | 124 |  | 7/7/04 11:56 | 80 | 16 | 27 | 399 | 35 |  | 8.5 |  |
| 8A | 27 |  |  |  |  | 35 |  |  |  | 8.5 | Attn. 6dB |
| 8A | 29 |  |  | 10 | 28 | 35 | 64 | 4 |  | 8.5 | Attn. 6dB |
| 8A | 30 |  |  | 40 | 28 | 35 | 117 | 9 | 9 | 8.5 | Attn. 6dB |
| 8A | 31 |  |  | 80 | 28 | 35 | 401 | 37 |  | 8.5 | Attn. 6dB |
| 8B | 133 |  | 7/8/04 2:30 |  |  | 28 |  |  |  | 8.5 | Appears scuffed. Used optical grease |
| 8B | 134 |  | 7/8/04 2:42 | 10 | 16 | 28 | 127 | 9 |  | 8.5 |  |
| 8B | 135 |  | 7/8/04 2:52 | 40 | 16 | 28 | 192 | 15 | 5 | 8.5 |  |
| 8B | 136 |  | 7/8/04 3:02 | 80 | 16 | 28 | 522 | 42 |  | 8.5 |  |
| 8C | 125 |  | 7/8/04 12:50 |  |  | 25 |  |  |  | 8.5 |  |
| 8C | 126 |  | 7/8/04 12:52 | 10 | 16 | 25 | 64 | 5 |  | 8.5 |  |
| 8C | 127 |  | 7/8/04 12:57 | 40 | 16 | 25 | 87 | 8 | 4 | 8.5 |  |
| 8C | 128 |  | 7/8/04 1:06 | 80 | 16 | 25 | 211 | 22 |  | 8.5 |  |
| 9A | 117 |  | 7/7/04 10:29 |  |  | 26 |  |  |  | 8.5 |  |
| 9A | 118 |  | 7/7/04 10:30 | 10 | 14 | 26 | 81 | 6 |  | 8.5 |  |
| 9A | 119 |  | 7/7/04 10:42 | 40 | 14 | 26 | 108 | 9 | 8 | 8.5 |  |
| 9A | 120 |  | 7/7/04 10:48 | 80 | 14 | 26 | 508 | 49 |  | 8.5 |  |
| 9B | 109 |  |  |  |  | 26 |  |  |  | 8.5 |  |
| 9B | 110 |  |  | 10 | 16 | 26 | 62 | 5 |  | 8.5 |  |
| 9B | 111 |  |  | 40 | 16 | 26 | 80 | 7 | 5 | 8.5 |  |
| 9B | 112 |  |  | 80 | 16 | 26 | 289 | 26 |  | 8.5 |  |
| 9C | 105 |  |  |  |  | 26 |  |  |  | 8.5 |  |
| 9C | 106 |  |  | 10 | 16 | 26 | 78 | 6 |  | 8.5 |  |
| 9C | 107 |  |  | 40 | 16 | 26 | 115 | 10 | 5 | 8.5 |  |
| 9C | 108 |  |  | 80 | 16 | 26 | 322 | 30 |  | 8.5 |  |
| 10A | 129 |  | 7/8/04 2:10 |  |  | 27 |  |  |  | 8.5 |  |
| 10A | 130 |  | 7/8/04 2:12 | 10 | 16 | 27 | 59 | 4 |  | 8.5 |  |
| 10A | 131 |  | 7/8/04 2:20 | 40 | 16 | 27 | 95 | 7 | 7 | 8.5 |  |
| 10A | 132 |  | 7/8/04 2:26 | 80 | 16 | 27 | 296 | 26 |  | 8.5 |  |
| 10B | 71 |  |  |  |  | 27 |  |  |  | 8.5 |  |
| 10B | 72 |  |  | 10 | 16 | 27 | 98 | 8 |  | 8.5 |  |
| 10B | 73 |  |  | 40 | 16 | 27 | 152 | 12 | 4 | 8.5 |  |
| 10B | 74 |  |  | 80 | 16 | 27 | 373 | 33 |  | 8.5 |  |
| 10C | 97 |  | 7/7/04 |  |  | 29 |  |  |  | 8.5 |  |
| 10C | 98 |  | 7/7/04 | 10 | 16 | 29 | 85 | 5 |  | 8.5 |  |
| 10C | 99 |  | 7/7/04 | 40 | 16 | 29 | 146 | 10 | 7 | 8.5 |  |
| 10C | 100 |  | 7/7/04 | 80 | 16 | 29 | 442 | 33 |  | 8.5 |  |

#### Table 2:  PE/CR ratio for UMSL7A to UMSL10C

##### Graphs

The following set of measurements were made after glueing a mirror on the bottom edge of the scintillator. The results are summarized in table 3

|  | Run # | Date / Time | Distance (cm) | Ped (Bin) | One Pe (bin) | CR | Pe/CR | Ratioof far(80cm) to near 10(cm) | Turns | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10A | 301 | 7/14/04 |  |  | 25 |  |  |  | 8.5 | The number of turns was kept constant. The low input power supply was disconnected while changing the sciltillarors. |
| 10A | 302 | 7/14/04 | 10 | 15 |  | 62 | 5 |  | 8.5 | PMT was found only loosely attached upon opening |
| 10A | 303 | 7/14/04 | 40 | 14 |  | 83 | 6 | 4 | 8.5 |  |
| 10A | 304 | 7/14/04 | 80 | 14 |  | 253 | 22 |  | 8.5 |  |
|  | 308 | 7/14/04 | 10 | 15 |  | 74 | 6 |  | 8.5 | Repositioned the PMT and sealed with tape. Positioning the PMT correctly is very important. In this case it increased the PE/CR ration by an average of 40% |
| 10A | 309 | 7/14/04 | 40 | 15 |  | 109 | 9 | 5 | 8.5 |  |
| 10A | 310 | 7/14/04 | 80 | 15 |  | 339 | 32 |  | 8.5 |  |
| 10B | 316 | 7/14/04 |  |  | 25 |  |  |  | 8.5 |  |
| 10B | 317 | 7/14/04 | 10 | 15 |  | 116 | 10 |  | 8.5 |  |
| 10B | 318 | 7/14/04 | 40 | 15 |  | 151 | 14 | 4 | 8.5 |  |
| 10B | 319 | 7/14/04 | 80 | 15 |  | 421 | 41 |  | 8.5 |  |
| 10C | 312 | 7/14/04 |  |  | 25 |  |  |  | 8.5 |  |
| 10C | 313 | 7/14/04 | 10 |  |  | 94 | 8 |  | 8.5 |  |
| 10C | 314 | 7/14/04 | 40 |  |  | 134 | 12 | 5 | 8.5 |  |
| 10C | 315 | 7/14/04 | 80 |  |  | 409 | 39 |  | 8.5 |  |
| 8A | 320 | 7/14/04 |  |  | 24 |  |  |  | 8.5 |  |
| 8A | 321 | 7/14/04 | 10 | 15 |  | 100 | 9 |  | 8.5 |  |
| 8A | 322 | 7/14/04 | 40 | 15 |  | 149 | 15 | 5 | 8.5 |  |
| 8A | 323 | 7/14/04 | 80 | 15 |  | 455 | 49 |  | 8.5 |  |

Table3: PE/CR values after mirror glued at the bottom of the scintillator.

Problems Encountered

The voltage to the PMT was turned to zero after testing each counter, so as not to suddenly jolt the PMT with a high voltage. However this posed a problem in resetting the voltage back to exactly the right voltage for each counter. Because of this we believe that the one PE measurement was changed for some of the counters tested. But because we measured the one P.E for each counter, this does not affect PE/CR or the near/far ratio.

###### Summary

The scintillators that will be used for the CosRay HS GPS time-stamped set-ups have been prepared. Initial tests showed PE (far end) to be between 4 and 8, but 20 to 40 at the end near the PMT, with near/far ratio typically being 5. Some improvement (20%) in near/far ratio was seen upon glueing a small piece of front surfaced mirror (aluminized polycarbonate originally used in E865 at BNL) to the far end of the scintillator, to redirect rays which would otherwise escape.

Work is continuing to prepare high gain bases for the PMTs that will be used in the final assembly. Final wrapping and testing are underway.